Assignment 1: Baseline Production Planning under Infinite Capacity

1.1 Assignment 1A: Production Optimization with Forecasted Demand under Infinite Capacity

To develop an optimal production plan under the assumption of unlimited production capacity, a Mixed Integer Programming (MIP) model is constructed. The objective of this model is to minimize the total cost, consisting of setup costs and inventory holding costs, over a 30-week planning horizon.

The model ensures that the forecasted demand for the final product (E2801) is satisfied while respecting the multi-stage structure of the Bill of Materials (BoM). This structure defines how intermediate components are assembled across different production stages. By incorporating lead times, minimum lot sizes, and component dependencies, the model provides a cost-efficient and feasible production schedule.

1.1.1 Sets

P = {B1401, B2302, B3201, B4702, V1501, V6302, E2801}, set of all parts
T = {1, 2, ..., 30}, set of time periods

1.1.2 Parameters

Lead times:

LTₚ = {
B1401: 4,
B2302: 2,
B3201: 1,
B4702: 5,
V1501: 3,
V6302: 2,
E2801: 1
}

Minimum lot sizes:

MLSₚ = {
B1401: 1200,
B2302: 1200,
B3201: 800,
B4702: 600,
V1501: 1500,
V6302: 700,
E2801: 600
}

Initial inventory:

I₀ₚ = {
B1401: 6400,
B2302: 4000,
B3201: 5200,
B4702: 3200,
V1501: 5800,
V6302: 1950,
E2801: 1350
}

Setup costs:

SCₚ = {
B1401: 2500,
B2302: 2500,
B3201: 6500,
B4702: 2500,
V1501: 2500,
V6302: 4000,
E2801: 4500
}

Holding costs:

HCₚ = {
B1401: 0.5,
B2302: 0.7,
B3201: 1.8,
B4702: 2.4,
V1501: 3.1,
V6302: 4.9,
E2801: 6.0
}

Forecasted demand for final product (E2801):

Dₜ = {
136, 144, 263, 123, 171, 195, 414, 147, 129, 245,
208, 178, 168, 208, 173, 168, 181, 188, 212, 253,
219, 196, 226, 239, 220, 199, 183, 149, 167, 205
}

1.4 Decision Variables

xₚ,ₜ ∈ ℝ₊ → production quantity of part p in period t

Iₚ,ₜ ∈ ℝ₊ → inventory level of part p at the end of period t

yₚ,ₜ ∈ {0,1} → binary variable indicating whether production of part p is set up in period t

1.5 Objective Function

The objective of this model is to minimize the total cost associated with production planning over the 30-week planning horizon. This total cost consists of two main components: setup costs and inventory holding costs.

Setup costs are incurred each time production is initiated for a specific part in a given period, while holding costs are incurred for storing inventory from one period to the next. As a result, the model aims to strike a balance between producing in larger batches to reduce the frequency of setups and minimizing excess inventory levels.

This trade-off is essential, as producing in large quantities reduces setup costs but increases holding costs, whereas frequent production reduces inventory levels but increases setup costs.

Minimize:

Min ∑ₚ∈P ∑ₜ∈T (SCₚ · yₚ,ₜ + HCₚ · Iₚ,ₜ)

1.6 Constraints

1.6.1 Inventory Balance with Lead Times and BoM Relations

The inventory balance constraints ensure that, for each part and each period, the available inventory is equal to the inventory from the previous period plus the production that becomes available after the corresponding lead time, minus demand.

For the final product (E2801), demand is external and given by the forecast. For intermediate components, demand is induced by the production of their parent parts according to the Bill of Materials (BoM).

1.6.1.1 Inventory Balance Constraints (for t = 1)

In the first period, no production has yet arrived due to lead times. Therefore, the inventory is determined by subtracting the immediate demand or component usage from the initial inventory levels.

For the final product:

IE2801,1 = I0E2801 − D1

For an intermediate component (example V1501):

IV1501,1 = I0V1501 − 4 · xE2801,1

For a lower-level component (example B1401):

IB1401,1 = I0B1401 − 4 · xV1501,1

1.6.1.2 Inventory Balance Constraints (for t = 2, ..., T)

From the second period onward, inventory evolves based on:

- the inventory from the previous period,

- the production that becomes available after the lead time,

- the external or induced demand.

The general inventory balance equation is:

Ip,t = Ip,t−1 + xp,t−LTp − demandp,t − induced demand

For the final product:

IE2801,t = IE2801,t−1 + xE2801,t−LT_E2801 − Dt

For an intermediate component:

IV1501,t = IV1501,t−1 + xV1501,t−LT_V1501 − 4 · xE2801,t

For a lower-level component:

IB1401,t = IB1401,t−1 + xB1401,t−LT_B1401 − 4 · xV1501,t

1.6.2 Minimum Lot Size Constraint

Each part has a predefined minimum lot size. This constraint ensures that whenever production is initiated, the produced quantity must be at least equal to the minimum lot size. This is achieved by linking the production variable to a binary setup variable.

xₚ,ₜ ≥ MLSₚ · yₚ,ₜ   ∀ p ∈ P, ∀ t ∈ T

1.6.3 Linking Setup to Production

To ensure that production only takes place when a setup occurs, an upper bound is imposed using a sufficiently large constant MM. This constraint guarantees that production is zero when no setup is performed.

xₚ,ₜ ≤ M · yₚ,ₜ   ∀ p ∈ P, ∀ t ∈ T

1.7 Results

The model was solved using Gurobi Optimizer and resulted in an optimal solution.

Total cost:

EUR 394,454

Breakdown:

- Setup cost: EUR 79,500

- Holding cost: EUR 314,954

1.8 Key Insights

- Holding costs represent the largest portion of the total cost, indicating that the model prefers fewer setups and larger production batches.

- The final product (E2801) contributes the most to the total cost due to its direct demand.

- Certain components, such as V6302, show zero holding cost, indicating just-in-time production behavior.

- The number of setups remains relatively low, confirming that the model effectively balances setup and holding costs.

1.2 Assignment 1B: Impact of Realized Demand and Backordering on Production Plan

To assess the impact of demand uncertainty in Assignment 1b, the production plan obtained in Assignment 1a was reused without re-optimization. The production quantities and setup decisions remained unchanged, while the realized demand was used instead of the forecasted demand for the final product E2801.

Since the intermediate components were produced according to the same production plan as in Assignment 1a, their setup-related costs remained unchanged. The inventory evolution of the final product E2801 was recalculated over the 30-week horizon by using the same production releases from Assignment 1a, taking into account the lead time and replacing forecasted demand by realized demand. Whenever the available inventory was insufficient to satisfy demand, the unmet quantity was recorded as a backorder and penalized at a cost of €250 per unit per period. This approach made it possible to evaluate the performance of the forecast-based production plan under actual demand conditions.

1.2.1 Comparison of KPIs for Assignment 1

The results show that the total cost increased from EUR 394,454 in Assignment 1a to EUR 447,792 in Assignment 1b. The setup cost remained unchanged at EUR 79,500, since the production plan itself was not modified. The total holding cost increased from EUR 314,954 to EUR 321,542, reflecting the updated inventory profile under realized demand. In addition, a backorder cost of EUR 46,750 was incurred, corresponding to 187 backordered units.

The service-related KPIs further illustrate the impact of demand uncertainty. Assignment 1b resulted in a service level of 93.33% and a fill rate of 96.89%. These results indicate that although the forecast-based production plan performs reasonably well under realized demand, demand deviations still lead to additional costs and some loss in service performance.